|  |  |  |  | Canyon CC 76- Vitreous China Rimless One-Piece Elongated Toilet with Skirted Trapway Floor Outlet, 1.28 gpf (4.8 lpf) with Seat |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

DIMENSIONS

COLOURS AND FINISHES

Gloss White

| Length | 710mm |
| --- | --- |
| Width | 400mm |
| Height | 730mm |
| Seat Height | 417mm |

CERTIFICATIONS & APPROVALS

FEATURES

| Flushing system | Rim Holes Flush, Dual flush |
| --- | --- |
| Nominal water consumption | 4.8L Full flush/ 4.1L Half flush |
| Material | Vitreous china |
| Rough-in | S-Trap 305mm |
| Flush button type | Buttons |
| Bowl Shape | Elongated |
| Seat height | 417mm |
| Lid Shape | Elongated, Urea-Formaldehyde (UF) Thermoset Plastic or PP , soft close, quick release |
| Trapway size | 45mm |

Images are for illustrative purposes only.

COMPATIBLE WITH

Dual Flush Buttons for

Canyon Toilet

Urea-Formaldehyde (UF) Thermoset Plastic or PP Soft Close Quick-Release, Easy-Clean lid for Canyon 76 Toilets

R&T FLUSH VALVE, DUAL FLUSH for

Canyon Toilet

R&T INFILL VALVE for

Canyon Toilet

TECHNICAL REQUIREMENTS

| Toilet type | One-piece Toilet |
| --- | --- |
| Installation Type | Floor-mounted |
| Water surface size | 206mm*140mm |
| Min static pressure | 138 kPa |
| Max static pressure | 552 kPa |
| Min flowing pressure | 172 kPa |
| Min flow rate | 1.6 lpm |
| Water Supply Location | ~203mm from floor to inlet, and 152mm – 203mm offset from centerline |

STANDARDS

ASME A112.19.2/CSA B45.1

U.S. EPA WaterSense® Specification for HETs (High-Efficiency Toilets)

U.S. Department of Energy (DOE) Energy Policy Act 1992

ADA / ICC ANSI A117.1 (when installed per code)

Certified by IAPMO R&T

California Energy Commission (CEC)

ICC/ANSI A117.1

CSA B651

KEY ADVANTAGES

- High-Efficiency 1.28 GPF Flushing System Consumes only 1.28 gallons per flush (approximately 4.8 litres), fully compliant with the EPA WaterSense® and cUPC certification standards—saving water while delivering strong flushing performance. - Skirted Trapway for Easy Cleaning The fully skirted design conceals the trapway, giving the toilet a sleek, modern look and making the exterior easier to clean. - Compact Elongated Bowl for Comfort and Room Saving The compact elongated seat is space saving and offers comfort to the users. - Standard 12" Rough-In Compatibility Designed with a 305 mm (12") rough-in, suitable for most residential and commercial bathroom installations in the U.S. market. - Soft-Close Thermoplastic Seat Included Comes with a quiet-close thermoplastic seat to prevent slamming, reduce noise, and increase durability during everyday use. - High-Quality Glazed Vitreous China Made of premium glazed ceramic, featuring a smooth, stain-resistant surface that ensures long-lasting durability and a polished appearance.

INCLUDED COMPONENTS

Standard Package Includes: - One-piece vitreous china toilet - Thermoplastic or PP soft-close toilet seat - Seat mounting hardware kit - Flush system set (fill valve + 3" flush valve) - Chrome-plated flush button - Floor mounting bolt kit - Wax ring seal - Decorative bolt caps (2 pcs) - Installation manual (English) Water supply line and angle stop valve are not included and must be purchased separately.