|  |  |  |  | Clarity CC 75- Vitreous China Rimless One-Piece Elongated Toilet with Skirted Trapway Floor Outlet, 1.28 gpf (4.8 lpf) with Seat |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

DIMENSIONS

COLOURS AND FINISHES

Gloss White

| Length | 27’’ |
| --- | --- |
| Width | 14-3/4’’ |
| Height | 30-1/8’’ |
| Seat Height | 16-7/16’’ |

CERTIFICATIONS & APPROVALS

FEATURES

| Flushing system | Rim Holes Flush, Dual flush |
| --- | --- |
| Nominal water consumption | 1.28 GPF Full flush/ 1.08 GPF Half flush |
| Material | Vitreous china |
| Rough-in | S-Trap 12’’ |
| Flush button type | Buttons |
| Bowl Shape | Elongated |
| Seat height | 16-7/16’’ |
| Lid Shape | Elongated, Urea-Formaldehyde (UF) Thermoset Plastic or PP, soft close, quick release |
| Trapway size | 1-3/4’’ |

Images are for illustrative purposes only.

COMPATIBLE WITH

Dual Flush Buttons for

Clarity Toilet

Urea-Formaldehyde (UF) Thermoset Plastic or PP, Soft Close Quick-Release, Easy-Clean lid for Clarity CC 75 Toilets

R&T FLUSH VALVE, DUAL FLUSH for

Clarity Toilet

R&T INFILL VALVE for

Clarity Toilet

TECHNICAL REQUIREMENTS

| Toilet type | One-piece Toilet |
| --- | --- |
| Installation Type | Floor-mounted |
| Water surface size | 8-1/8“ x 5/1/2” |
| Min static pressure | 20 psi |
| Max static pressure | 80 psi |
| Min flowing pressure | 25 psi |
| Min flow rate | 0.43 gpm |
| Water Supply Location | ~8’’ from floor to inlet, and 6’’ – 8’’ offset from centerline |

STANDARDS

ASME A112.19.2/CSA B45.1

U.S. EPA WaterSense® Specification for HETs (High-Efficiency Toilets)

U.S. Department of Energy (DOE) Energy Policy Act 1992

ADA / ICC ANSI A117.1 (when installed per code)

Certified by IAPMO R&T

California Energy Commission (CEC)

ICC/ANSI A117.1

CSA B651

KEY ADVANTAGES

- High-Efficiency 1.28 GPF Flushing System Consumes only 1.28 gallons per flush, fully compliant with the EPA WaterSense® and cUPC certification standards—saving water while delivering strong flushing performance. - Skirted Trapway for Easy Cleaning The fully skirted design conceals the trapway, giving the toilet a sleek, modern look and making the exterior easier to clean. - Elongated Bowl for Superior Comfort The elongated seat offers more space and better ergonomics than round-front bowls, enhancing user comfort during use. - Standard 12" Rough-In Compatibility Designed with a 12" rough-in, suitable for most residential and commercial bathroom installations in the U.S. market. - Soft-Close Thermoplastic Seat Included Comes with a quiet-close thermoplastic seat to prevent slamming, reduce noise, and increase durability during everyday use. - High-Quality Glazed Vitreous China Made of premium glazed ceramic, featuring a smooth, stain-resistant surface that ensures long-lasting durability and a polished appearance.

INCLUDED COMPONENTS

Standard Package Includes: - One-piece vitreous china toilet - Thermoplastic or PP soft-close toilet seat - Seat mounting hardware kit - Flush system set (fill valve + 3" flush valve) - Chrome-plated flush button - Floor mounting bolt kit - Wax ring seal - Decorative bolt caps (2 pcs) - Installation manual (English) Water supply line and angle stop valve are not included and must be purchased separately.